LAB 5 – VELOCITY CONTROL OF A DC BRUSHED MOTOR

University of New Hampshire

30. November 2017

Jesse Feng
Simon Popecki
Reilly Webb

VELOCITY CONTROL OF A BRUSHED DC MOTOR

| Course Number and Name: ME 747 – Senior Lab |  |
| --- | --- |
| Semester and Year: 2017 semester 2 | Name of Lab Instructor: Alireza Ebadi |
| Lab Section and Meeting Time: 2b 14:00 | Report Type: External Group Report |
| Title of Experiment: Velocity Control of a DC Brush Motor |  |
| Date Experiment Performed: 14. November 2017 | Date Report Submitted: 1 December 2017 |
| Names of Group Members: Jesse Feng Simon Popecki Reilly Webb | Grader's Comments: |
|  | Grade: |

# Cover Letter

Jesse Feng
Simon Popecki
Reilly Webb
33 Academic Way
Durham, NH 03824

Alireza Ebadi
33 Academic Way
Durham, NH, 03824

Dr. Ebadi,

The following document contains an analysis of control systems for a DC brushed motor. DC motors are generally controlled via pulse width modulation and a microprocessor, however for the purpose of demonstration, we controlled DC motors with power op-amp driven proportion control, integral control, and proportional-integral control.

These control systems were compared against each other in terms of functionality- the motor parameters have been determined through experimentation.

The body of this report comprises of the results of inputs to the system and recorded system response.

Best Regards,

Jesse Feng

Simon Popecki

Reilly Webb

# Contents

Cover Letter	3

Contents	4

Objectives	5

Executive Summary	6

Theory and Experimental Methods	7

Results and Discussion	12

Conclusions	13

Appendix	14

# Objectives

The objective of this experiment was to compare different methods of control using a power operational amplifier. Proportional, Integral, and Proportional-Integral control systems controlled a motor under load to be measured by a second motor.  The parameters of the motor were calculated, and the system response determined from experimental data. The motor back EMF, open-loop, and closed-loop response were analyzed in particular. Quantitative analysis was performed and interpreted. All relevant values are tabulated/listed. Gain values were confirmed by root locus analysis.

# Executive Summary

In this experiment three types of power op-amp control systems were used to control the speed of a DC brushed motor. Proportional control, Integral control, and Proportional-Integral control systems were used. PID control was not used in this experiment. Motors were controlled by voltage, rather than the conventional method of pulse width modulation. The objective of the control systems was to maintain motor speed regardless of the load placed on the motor. Motor speed was measured by a tachometer.

This experiment assumes that the electrical time constant was much less than the mechanical time constant – so that it may be neglected in the analysis.

The DC motor’s back EMF constant was found to be #. This value was determined by back driving another motor, referred to in this report as the MUT (motor under test). The torque constant Kt was calculated to be # - it was assumed that Kt = Ke. The resulting motor torque value was calculated to be # oz*in/A. This value matches the specifications provided by the motor manufacturer.

The motor’s open-loop response for a disturbance torque and input voltage step was recorded and analyzed. The time constant and system gain were calculated – a disturbance load was implemented with a switch to measure steady state error – the steady state error is the difference between the motor’s no-load speed and speed under load. The motor time constant was found to be #, and the motor gain value was found to be #. The stall torque of the motor was calculated to be #. The moment of inertia and damping constant of the system were calculated to be # and # respectively.

Closed-loop motor response to a voltage step input and disturbance torque was recorded with a tachometer in the same way as the open-loop response. The power op-amps were controlled by a 741 op-amp – proportional, integral, and proportional-integral control were implemented and analyzed. SHOW SPEEDS OF EACH CONTROL SYSTEM. In a purpose-built closed-loop system the magnitude of each control element (P,I,D) is modified such that it has a larger or smaller role to fit the individual needs of the system. Generic control parameters were used for this experiment (non-optimized).

# Theory and Experimental Methods

Figure 1

The experimental setup for the open-loop response is shown in Figure 1 below.

The motor left most is the MUT – the motor under test, and the motor on the right is the motor which is back driven and used to provide a load for the MUT. The two motors are coupled with a flex-coupling to allow for minor shaft misalignment. The MUT was supplied with power from a power op-amp. The MUT was connected to a tachometer, which outputted a voltage proportional to the rotational velocity.

Figure 2

Figure 2 below shows the schematic describing the power op-amp setup.

The gain of the power op-amp is -1, which means the motor spins the opposite direction (not relevant to the results). Voltages of +15 and -15 volts. The power op-amp was supplied by BK Precision, it provides the power for the motor. Both resistors are 100 kΩ.

Figure 3

The open-loop block diagram of the motor is shown below in Figure 3.

The time constant of the electrical system is much shorter than the time constant of the mechanical system, it is therefore neglected. In this case, the mechanical pole is dominant.

Figure 4: DC Brush Motor Electro-mechanical model

1.1 DC Motor Systems Modeling

(Equation 1)

The electro-mechanical model of a DC brush motor is shown in figure 4. The electrical circuit takes an input voltage and generates a voltage drop ed across the rotor. The electrical energy from the voltage drop can be related to the resulting angular velocity, ω, through the motor velocity constant, Ke:

ed=Keω

(Equation 2)

The current of the circuit, i, can also be coupled to the resulting torque in the motor, T, through the back-EMF constant, Kt:

T=Kti

(Equation 3)

Performing electrical and mechanical analysis on the system using equations 1 and 2 leads to the following transfer function relating the output angular velocity to the input voltage:

Gmotors=ΩEa=KtJRs+BR+KtKe

Equation 3 is typically written in the following format for ease of identifying the system parameters, system gain, K, and time constant, τ:

Gmotors=ΩEa=Kτs+1=KtBR+KtKeJRBR+KtKes+1

(Equation 5)

(Equation 4)

Therefore, the equations for the system gain and time constant of a motor are:

K=KtBR+KtKe

τ=JRBR+KtKe

The detailed derivation steps for the motor transfer function are attached in Appendix A1.1 in this document.

Figure 5: Experimental Setup Block Diagram

1.2 Experimental Setup and Methods

The full experimental setup is shown in the block diagram in figure 5. The generated input voltage ei is passed through a power op-amp with a gain of Rf/Ri=1 to produce the motor input voltage ea. The motor’s resulting angular velocity is fed back to complete a closed-loop feedback system through the motor velocity constant. The difference (error) between the input voltage and the output angular velocity controls the motor. The 1/(Ls+R) term represents the electrical system’s resulting voltage drop at the rotor, which converts the electrical energy into torque through the back-EMF constant. Potential disturbance loading is included in the schematic. The angular velocity is measured by a tachometer with a gain of Ktach and outputs a voltage measurable with a scope.

It is worth noting that when the disturbance load is equal to the generated torque from a given voltage, Td=T, the input into the mechanical model 1/(Js+B) would be zero, resulting in zero angular velocity. But the motor is still generating a torque T. This torque is the stall torque at the given voltage Td=T=Tstall.

A back-drive motor is connected to the motor under test (MUT). In the first part, the back-drive motor delivers torque to the MUT, converting the motor into a generator, and the generated voltage and the tachometer output for the back-drive motor are measured. The data is used to determine the back-EMF constant, Ke, of the MUT by linearly fitting the angular velocity with the MUT output, assuming the loss between the back-drive and MUT connection is negligible.

In the second part, after Ke is determined, the MUT is directly connected to determine the motor’s time constant, gain, and response to disturbance load. The steady-state error of the disturbance response is the difference between the output before and after engaging the disturbance.

(Equation 6)

Finally, the system is then connected to proportional, integral, and proportional-integral controllers to form a closed-loop system. The three controllers’ circuit diagram is shown in figure 6 (next page). The P, I, and PI controllers’ transfer functions are shown in equations 6 to 8. The P controller’s gain is 1.5:

Gc,P=EiEω=RfRi=1.5

(Equation 8)

(Equation 7)

Gc,  I=EiEω=1RiCis

Gc,PI=EiEω=RfC2s+1RiC2s

Figure 6: P, I, and PI Controller circuit diagrams showing connection to the rest of the system

# Results and Discussion

Figure 7 – MUT output voltage vs rotational speed

The back-EMF constant was found by driving the MUT with varying voltages, and measuring the resulting RPM. The resulting curve is a linear response, which is used to find the motor’s torque constant, Kt. The curve is shown in Figure 7 below.

The motor voltage constant was found using the slope of the line in Figure 7. This was used to find the motor torque constant. Kt was found to be 6.7273 Oz-in/A which is what the manufacturer specified in their publication. Ke , the motor voltage constant was found to be 4.9817 volts per thousand RPM.

# Conclusions

# Appendix

A1.1 DC Motor Electromechanical Model Derivation

Given the following system for a DC motor:

From the circuit, perform Kirchhoff’s loop law:

ea-Ri-Ldidt-ed=0

ea=ed+Ldidt+Ri

The voltage drop ed is proportional to the resulting angular velocity of the mechanical system, and the current is proportional to the resulting torque by:

ed=Keω, T=Kti

Using JMUT+JBK=J and BMUT+BBK=B, the mechanical system equation is:

Jω+Bω=T=Kti

Since the inductance is three magnitudes less than the resistance, the inductance term is neglected. Laplace transform and combine the equations:

JΩs+BΩ=KtEa-KeΩR

JRΩs+BRΩ=KtEa-KtKeΩ

JRs+BR+KtKeΩ=KtEa

The transfer function relating output angular velocity to input voltage is then:

Gmotors=ΩEa=KtJRs+BR+KtKe

A1.2 Controller transfer function derivations

Given the P controller setup shown in figure 3, assuming ideal op-amp, performing nodal analysis on the negative lead node gives:

Eω-E-Ri+E--EiRf=0

Ideal op-amp means e-=e+, and e+ is grounded to zero. Therefore:

EωRi+-EiRf=0

Rearranging:

Gc,P=EiEω=RfRi

Similarly, for I controller:

Eω-E-Ri+C1s(E--Ei)=0

EωRi+C1s(-Ei)=0

Gc,I=EiEω=1RiC1s

The PI controller is slightly more complicated. The resistor-capacitor in series need to be combined into one element:

Rf+1C2s=>RfC2s+1C2s

The repeat the same steps:

Eω-E-Ri+RfC2s+1C2s(E--Ei)=0

EωRi=RfC2s+1C2sei

Gc,PI=EiEω=RfC2s+1RiC2s